North Carolina Division of Public Health Guidance: Personal Protective Equipment (PPE) in Hospital Ebola Isolation Units

Doffing Checklist

Date: ____/____/______								Designated Site Monitor Name: ________________________________

Healthcare Personnel Name: __________________________________

Instructions: This checklist is to be completed by the designated site monitor for every healthcare worker that accesses the Ebola isolation patient care area. It is critical that each step be completed correctly and documented. The designated site monitor should also wear the following PPE during the doffing process: 1) Single-use impermeable gown or coverall; 2) Single-use full face shield; 3) Two pair of single-use nitrile examination gloves; the outer pair should have extended cuffs.

| Doffing Checklist Item | Description | Step performed appropriately? | If No, or Breach identified, explain | Site Monitor Initials |
| --- | --- | --- | --- | --- |
| - While in patient room: - Hand hygiene on outer gloves - Inspect and wipe PPE - Hand hygiene on outer gloves - Assure designated site monitor is present and engaged | - Perform hand hygiene on outer gloves. - Inspect PPE (with buddy, if present) and us an EPA-approved disinfectant wipe to clean PPE. - Perform hand hygiene once more. - Engage the monitor to observe and assist with doffing; only 1 healthcare personnel should exit and doff at a time. | ☐ Yes ☐ No |  |  |
| - Step into chlorine bath | Chlorine bath should be used with 1:10 or 0.5% bleach solution and should cover top of boots. | ☐ Yes ☐ No |  |  |
| - Step to absorbent walk-off mat | Step onto designated walk-off mat, stay in this area for doffing. | ☐ Yes ☐ No |  |  |
| - Perform hand hygiene on double-gloved hands | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Inspect PPE/wipe visible contamination | All PPE should be inspected for any visible breaches. Document breaches and notify appropriate Occupational Health & Safety personnel per hospital protocol. | ☐ Yes ☐ No ☐ Breach identified (explain) |  |  |
| - Perform hand hygiene on double-gloved hands | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Remove outer gloves | Use glove-in-glove technique to remove outer gloves. Dispose. | ☐ Yes ☐ No |  |  |
| Doffing Checklist Item | Description | Step performed appropriately? | If No, or Breach identified, explain | Site Monitor Initials |
| - Inspect inner gloves | Inspect remaining inner gloves for any visible breaches. If no breaches identified: Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. If breaches identified: 1) Perform hand hygiene on gloved hands, 2) Remove inner gloves, 3) Perform hand hygiene on bare hands, 4) Don clean gloves | ☐ Yes ☐ No ☐ Breach identified (explain) |  |  |
| - Perform hand hygiene on inner gloves | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Remove apron | Gently remove and roll from the inside out to avoid contaminating self and environment. Dispose. | ☐ Yes ☐ No ☐ N/A |  |  |
| - Perform hand hygiene on inner gloves | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Inspect/wipe visible contamination (especially on torso) | Following apron removal, use EPA-approved disinfectant wipe to wipe down visible contamination, particularly on torso. Dispose of wipe. | ☐ Yes ☐ No ☐ Breach identified (explain) |  |  |
| - Perform hand hygiene on inner gloves | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Remove gown or coverall | Gown: Untie/unfasten, or receive assistance. Avoid contact with scrubs during removal, pull away from body and roll inside out. Only touch inside of gown. Dispose. Coverall: Tilt head back, reach under hood without touching sleeves to head or face. Unzip completely, pull out and away from body and down, then roll inside out. Avoid contact with scrubs. Dispose. | ☐ Yes ☐ No |  |  |
| - Perform hand hygiene on inner gloves | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Remove face shield | Lean forward, tilt head forward, grab rear strap, pull over head until it falls forward. Do not touch surface of shield. Dispose. | ☐ Yes ☐ No |  |  |
| - Perform hand hygiene on inner gloves | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Remove surgical hood | Lean forward, pull up and overhead or unfasten to remove, and discard. Dispose. | ☐ Yes ☐ No |  |  |
| - Perform hand hygiene on inner gloves | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| Doffing Checklist Item | Description | Step performed appropriately? | If No, or Breach identified, explain | Site Monitor Initials |
| - Inspect/wipe plastic boots | Use EPA-approved disinfectant wipe to wipe down visibly contaminated boots. Dispose of wipe. | ☐ Yes ☐ No ☐ N/A |  |  |
| - Perform hand hygiene on inner gloves | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Remove inner gloves | Use glove-in-glove technique to remove outer gloves. Dispose. | ☐ Yes ☐ No |  |  |
| - Perform hand hygiene on bare hands | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Don clean pair of gloves | Don clean pair of nitrile gloves before removing N95. | ☐ Yes ☐ No |  |  |
| - Perform hand hygiene on gloves | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Remove N95 respirator | Tilt head forward, grasp bottom strap on back of head, then grasp top strap; remove without touching front of N95. Dispose. | ☐ Yes ☐ No |  |  |
| - Perform hand hygiene on inner gloves | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Remove inner gloves | Use glove-in-glove technique to remove. Dispose. |  |  |  |
| - Perform hand hygiene on bare hands | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Inspect healthcare personnel | Inspect healthcare worker for any visible contamination or breaches. | ☐ Yes ☐ No ☐ Breach identified (explain) |  |  |
| - Remove boots | While sitting down, pull off one boot at a time, keeping the boots in the contaminated section of the doffing area. Once removed from boots, feet should be placed and remain in the clean section of the doffing area. | ☐ Yes ☐ No |  |  |
| - Perform hand hygiene on bare hands | Use soap and water, EPA-approved disinfectant wipe, or alcohol-based hand rub. | ☐ Yes ☐ No |  |  |
| - Put on shoes, change scrubs, and shower | Healthcare worker should put on shoes, change out of current scrubs, shower and don clean scrubs or clothes. | ☐ Yes ☐ No |  |  |
| - Rehydrate/replenish | Assure nutrition and hydration of healthcare worker. | ☐ Yes ☐ No |  |  |